General Instructions for Replacing Damaged Glass Assembly for Kenyon UL Knob Control Cooktops

Replacement Glass Disclaimer:

By supplying these instructions Kenyon does not assume any liability for work performed on a Kenyon product.   Kenyon International, Inc accepts no responsibility for product liability or proper operation of this stove after it has been disassembled and reassembled by anyone other than a Kenyon International, Inc employee. Only factory trained, certified appliance technicians should make such a repair.

- Remove knobs from stems of controllers by simply lifting off switch shafts.

- Place cooktop glass side down on a workable surface (i.e. flat table).

- Using a screwdriver, loosen the locking screw on the conduit connector mounted to the bottom of the cooktop.  Slide the conduit down the wires until completely off.  Lay to the side.

- Using a Phillips head screwdriver, remove sheet metal screws that are holding the bottom pan cover to the cooktop.  Store all screws nearby.  Carefully lift cover off the stove guiding the wire pigtail through the hole in the cover.

- Remove sheet metal screws holding the burner mounting brace to the metal frame attached to the glass.  DO NOT lift off the burner brace/burners at this point.

- Remove sheet metal screws that are holding the switch panel compartment to the metal frame attached to the glass.

- Carefully lift the burner brace/burners and switch panel compartment off the damaged glass surface being careful of wires on bottom of cooktop.  DO NOT disconnect any wires.  Place burner brace/burners and switch panel compartment aside.  Be very careful handling burner brace and switch compartment.  DO NOT damage components. Inspect burners and switches for any visible damage.

- Carefully remove rubber grommets from the damaged glass panel. Discard damaged glass panel.  Install rubber grommets carefully into replacement glass panel.  Before proceeding place something “soft” over work area that will prevent glass scratching on new panel.  These items could include bubble wrap, soft linen tablecloth, protective foam film, bath towel, etc.  Place the replacement glass panel (cooking side down) on this surface.

- Carefully place burner brace/burner(s) and switch panel compartment into the metal frame attached to the replacement glass panel.  Make sure switch shafts pass through rubber grommet openings.  Install plastic knobs back onto switch shafts.  Move the switch panel compartment until knobs appear to be centered with control graphics.  Install and tighten sheet metal screws into the switch plate.

- Install and tighten screws back into the burner brace.

- Replace bottom pan cover. Fasten bottom pan back to the metal frame using the removed sheet metal screws.  Carefully guide wire cord through the hole opening then through conduit and tighten conduit connector.  Replace conduit locking plate and tighten screw.  Your cooktop is now ready to be reinstalled into countertop.

If you have any questions, please feel free to contact our Technical Support Department @ 860-664-4906.